# DESIGN LAYOUT

sTARS Integrated Food Ecosystem

Mars Surface Configuration — 15 POD Architecture

Team: Bueché-Labs LLC
Deep Space Food Challenge: Mars to Table
Version 4.0 — May 2026

# 1. Overall Habitat and System Layout

The sTARS Integrated Food Ecosystem comprises 15 modular PODs connected to a central spine structure, achieving 90% Earth-independence for a 15-person crew over 500+ sols.

[Figure 1: Site Layout — 15 PODs on Central Spine with SEP Protection]

## 1.1 Volume Justification

| Component | Qty | Volume | Function |
| --- | --- | --- | --- |
| Food PODs 1-5 | 5 | 2,275 m³ | Human crop production + oilseeds |
| Food POD 6 (Fodder) | 1 | 455 m³ | Livestock fodder |
| Food POD 7 (Grain) | 1 | 455 m³ | Grain production |
| Livestock POD | 1 | 455 m³ | Goats + chickens |
| Aquaponics POD (NEW) | 1 | 455 m³ | Tilapia fish farming |
| Food Processing POD (NEW) | 1 | 455 m³ | Oil, fermentation, milling, drying |
| RSV PODs | 2 | 910 m³ | Water extraction, power |
| Nutrient Processing | 1 | 455 m³ | Haber-Bosch, urine/manure |
| Waste Processing | 1 | 455 m³ | Anaerobic digestion, biogas |
| HAB/LAB (Kitchen) | 1 | 455 m³ | Cooking, dining, storage |
| TOTAL FOOD SYSTEM | 15 | 6,825 m³ | 90% Earth-independent |

POD dimensions: 10.0m length × 7.6m diameter. Volume per POD: π × (3.8)² × 10 = 455 m³

# 2. Resource Management

## 2.1 Water Source and Recycling

| Component | Specification |
| --- | --- |
| Primary Source | Mars subsurface ice extraction via RSV PODs (Rodriguez-style well) |
| Extraction Rate | 1,400 L/day capacity (700 L × 2 RSV PODs) |
| Filtration System | Multi-stage: sediment → activated carbon → UV → RO membrane |
| Recycling Efficiency | 95% recovery from crops, crew, livestock, aquaponics |
| Aquaponics Loop | 8,000 L closed system; 5% daily makeup water (NEW) |
| Storage Capacity | 14-day reserve + distributed POD wall shielding (800 L/POD) |
| Backup | Dual RSV PODs (N+1 redundancy); H₂ combustion emergency |

## 2.2 Power Supply and Storage

| Component | Specification |
| --- | --- |
| Primary Source | iROSA solar arrays: ~450 kW average daytime |
| Secondary Source | RSV regenerative fuel cells (H₂/O₂): 100 kW backup (2 × 50 kW) |
| Supplemental Source | Biogas SOFC (Waste POD): 3-5 kW continuous |
| Food System Load | 385 kW total (see breakdown below) |
| Energy Storage | RSV H₂ tanks: 72-hour backup at reduced operations |
| Failsafe Mode | Priority load shedding: Life support → Livestock → Aquaponics → Crops |

## 2.3 Power Budget Breakdown

| System | Load (kW) |
| --- | --- |
| Food PODs 1-5 (LEDs, pumps) | 150 |
| Food PODs 6-7 (Fodder, Grain) | 60 |
| Livestock POD | 15 |
| Aquaponics POD (NEW) | 25 |
| Food Processing POD (NEW) | 20 |
| RSV PODs (×2) | 50 |
| Nutrient Processing | 30 |
| Waste Processing | 15 |
| HAB/LAB | 20 |
| TOTAL | 385 |

# 3. Control and Operations

## 3.1 Sensor Network and Automation

| System | Sensors | Automation Response |
| --- | --- | --- |
| Atmosphere | O₂, CO₂, temp, humidity | Auto-adjust HVAC, CO₂ injection |
| Hydroponics | pH, EC, flow rate, level | Auto-dose nutrients, adjust pH |
| Aquaponics (NEW) | Temp, pH, ammonia, nitrate, DO | Auto-feed, heater control, alerts |
| Fermentation (NEW) | Temp, pH, pressure | Climate control, pH monitoring |
| Lighting | PAR, spectrum, duration | Photoperiod control per crop stage |
| Livestock | RFID, cameras, scales | Feed/water auto-dispense, health monitoring |

## 3.2 Robotic Systems (Optimus Fleet)

| Assignment | Units | Tasks |
| --- | --- | --- |
| Food PODs 1-5 | 3 | Planting, harvesting, pruning, cleaning, transport |
| Food PODs 6-7 | 1 | Fodder/grain harvesting, milling support, transport |
| Livestock POD | 1 | Feed distribution, water refill, waste collection |
| Aquaponics POD (NEW) | 1 | Fish feeding, water testing, tank cleaning |
| Food Processing POD (NEW) | 1 | Oil press, fermentation, milling assist |
| Spare/Charging | 2 | Rotation; backup; surge capacity |
| TOTAL | 10 | Reduces crew labor by ~65% |

# 4. Production Systems

[Figure 2: Food POD — Vertical Farming with LED Arrays and Hydroponic Channels]

## 4.1 Growing Units and Configuration

| POD | Growing Area | System Type | Primary Crops |
| --- | --- | --- | --- |
| Food 1-5 (×5) | 361 m² each | Vertical hydroponic | Potatoes, vegetables, legumes, oilseeds |
| Food 6 (Fodder) | 361 m² | Fodder sprouting | Barley sprouts, alfalfa, beets |
| Food 7 (Grain) | 361 m² | Soil-less grain | Wheat, amaranth, buckwheat |
| Aquaponics (NEW) | 50 m² grow beds | Aquaponic NFT | Lettuce, herbs, greens (fish-fertilized) |
| TOTAL | 2,577 m² |  | Output: 40,975 kcal/day in-situ |

# 5. Aquaponics POD (NEW)

[Figure 5: Aquaponics POD — Fish Tanks (Deck 1), Biofilter (Deck 2), Grow Beds (Deck 3)]

## 5.1 System Layout

| Deck | Components | Function |
| --- | --- | --- |
| Deck 1 | 4 × 2,000L fish tanks, aeration, heaters | Tilapia grow-out and broodstock |
| Deck 2 | Biofilter, settling tank, pumps | Ammonia → Nitrate conversion |
| Deck 3 | 50 m² NFT grow beds, harvest area | Plant production + fish processing |

## 5.2 Fish Production Specifications

| Parameter | Specification |
| --- | --- |
| Species | Nile Tilapia (Oreochromis niloticus) |
| Total Volume | 8,000 L (4 tanks × 2,000 L) |
| Population | 200+ fish at various growth stages |
| Stocking Density | 50 kg/m³ (conservative) |
| Broodstock | 10 adults; 21-day spawning cycle; ~200 fry/spawn |
| Growth Rate | 3 g/day at optimal conditions |
| Market Weight | 500 g (180 days from fry) |
| Daily Harvest | 0.5-1 kg (2-3 fish) |
| Protein Output | 100-200 g protein/day |
| Calories | 525-1,050 kcal/day |

# 6. Food Processing POD (NEW)

[Figure 6: Food Processing POD — Oil Press (Deck 1), Fermentation (Deck 2), Drying/Milling (Deck 3)]

## 6.1 Oil Extraction Equipment

| Equipment | Capacity | Output |
| --- | --- | --- |
| Cold Press (mechanical) | 10 kg seeds/hour | 2-3 L oil/day |
| Oil Filter | Continuous | Food-grade clarity |
| Meal Collector | 8-10 kg/day | Protein meal for livestock |
| Storage Tanks | 50 L | Dark, cool, sealed |

## 6.2 Fermentation Equipment

| Equipment | Capacity | Products |
| --- | --- | --- |
| Fermentation Vessels (×4) | 20 kg each | Sauerkraut, kimchi, tempeh, miso |
| Temperature Controller | 18-32°C range | Product-specific settings |
| Yogurt Incubator | 5 L batch | Daily yogurt from goat milk |
| Cheese Cave | 12°C, 85% RH | Aged cheese maturation |

## 6.3 Grain Milling and Food Drying

| Equipment | Capacity | Products |
| --- | --- | --- |
| Stone Burr Mill | 20 kg/hour | Wheat, rice, corn, sorghum flour |
| Flour Sifter | Adjustable mesh | Whole grain or refined |
| Food Dehydrator | 10 kg fresh/batch | Dried fruits, vegetables, herbs |
| Vacuum Sealer | Chamber type | Long-term storage packaging |

# 7. Livestock POD

[Figure 7: Livestock POD — Goats (Deck 1), Dairy Processing (Deck 2), Chickens (Deck 3)]

## 7.1 Livestock Inventory and Output

| Animal | Count | Daily Output | Products |
| --- | --- | --- | --- |
| Nigerian Dwarf Does | 6 | 8 L milk | Milk, cheese, yogurt, whey |
| ISA Brown Hens | 20 | 17 eggs | Fresh eggs (1.1/crew/day) |
| Buck + Roosters | 3 | — | Breeding / herd sustainability |
| Kids + Chicks (rotating) | 10-18 | ~550 g meat/wk | Meat + herd/flock replacement |

# 8. Circular Resource System

[Figure 8: Closed-Loop Resource Flow — Including Aquaponics and Food Processing]

## 8.1 Resource Recovery Rates

| Resource | Recovery | Method |
| --- | --- | --- |
| Water | 95% | Transpiration, urine, greywater, aquaponics recirculation |
| Nitrogen | 90% | Haber-Bosch (Mars N₂) + manure + fish waste composting |
| Phosphorus | 80% | Struvite precipitation from urine + manure + fish solids |
| Carbon | 85% | CO₂ → plants → O₂ → crew → CO₂; biogas capture |
| Organic Waste | 95% | Anaerobic digestion → biogas + digestate fertilizer |
| Fish Waste (NEW) | 100% | Direct to plant nutrients via aquaponics loop |
| Oil Meal (NEW) | 100% | Protein-rich livestock feed supplement |

# 9. System Output: Daily Production

[Figure 9: Mars-Grown Meal — Including Fresh Fish, Fermented Foods, and Cooking Oil]

| Product | Daily Output | Per Crew | Calories |
| --- | --- | --- | --- |
| Fresh Vegetables | 15-20 kg | 1.0-1.3 kg | ~8,000 kcal |
| Potatoes/Starches | 8-10 kg | 500-650 g | ~18,800 kcal |
| Flour (grain) | 5.5 kg | 365 g | ~4,500 kcal |
| Fresh Eggs | 17+ eggs | 1.1 eggs | ~1,275 kcal |
| Goat Milk + Cheese | 8 L + 300 g | 530 ml + 20 g | ~5,300 kcal |
| Tilapia Fish (NEW) | 0.5-1 kg | 33-66 g | ~750 kcal |
| Vegetable Oil (NEW) | 2-3 L | 130-200 ml | ~2,200 kcal |
| Fermented Foods (NEW) | Variable | Variable | ~150 kcal |
| IN-SITU TOTAL | — | — | ~40,975 kcal (90%) |
| Earth-Supplied | Variable | Variable | ~4,550 kcal (10%) |
| TOTAL | — | 3,035 kcal | 45,525 kcal/day |

90% Earth-Independence — 40 points above the 50% requirement

— END OF DOCUMENT —